\begin{abstract}

This work reports on the derivation and analytic solution method for free in-plane modal solution for isotropic circular plate. The circular shell plate is well studied and the modal solution based on Bessel function is well documented. However the in-plane behavior is not well studied as these modes occur at very high frequency and not generally important in the study of elastic behavior of circular plates. In this report the theoretical formulation, governing partial differential equation is derived in detail from first principles. The analytic solution is studied in detail. The analytic solution is solved using superposition technique.

\end{abstract}

\section{Introduction}

The in-plane vibration modes of circular plates is evaluated starting with the stress strain relationship formulation of 2D infinitesimal element due to the displacement field. This is used to find the dynamic equilibrium partial differential equation. First the equations are derived in rectangular coordinates and then its transformed to polar coordinates. The detailed Bessel function solution is then derived. The solution results are presented by implementing in OpenGL c++ code.

\section {Formulation of }

In order to study the inplane behavior of the circular section, we start with the derivation of the elemental strain tensors which will eventually be used to derive the equilibrium equations governing the in-plane vibration behavior. The below figure \ref{fig1_controlvolume_deform} shows the 2D infinitesimal element transformed due to the displacement field experienced by the element.

\begin{figure}[H]

\centering

\includegraphics[width=0.8\textwidth]{collision_response_pictures/pic1_deformation_controlvolume.png}

\caption{Deformation of control volume a) Deformation in x axis, b) Deformation in y axis.}

\label{fig:fig1_controlvolume_deform}

\end{figure}

Using the Taylor series expansion, the displacement field can be derived as,

ux+∆x,y=ux,y+∂u∂xdx

ux,y+∆y=ux,y+∂u∂ydy

vx,y+∆y=vx,y+∂v∂ydy

vx+∆x,y=vx,y+∂v∂xdx

In figure, we show an infinitesimal element used to represent the general two-dimensional state of strain at some point in a structure. The element is shown to be displaced by amounts u and v in the x and y directions at point A, and to displace or extend an additional (incremental) amount (ðu/ðx) dx along line AB, and (ðv/ðy) dy along line AC in the x and y directions, respectively. Furthermore, observing lines AB and AC, we see that point B moves upward an amount (ðv/ðx) dx with respect to A, and point C moves to the right an amount (ðu/ðy) dy with respect to A.

From the general definitions of normal and shear strains and the use of Figure, we obtain

ϵxx=∂u∂x

ϵyy=∂v∂y

γxy=∂u∂y+∂v∂x

In the matrix operator format for 2D case, the strain – displacement relation are

ϵxxϵyyγxy=∂∂x00∂∂y∂∂y∂∂xuv

\subsection{Constitutive Equation}

The strains ex and ey are the changes in length per unit length of material fibers originally parallel to the x and y axes, respectively, when the element undergoes deformation. These strains are then called normal (or extensional or longitudinal) strains. The strain gxy is the change in the original right angle made between dx and dy when the element undergoes deformation. The strain gxy is then called a shear strain.

I now present the stress/strain relationships for isotropic materials for both plane stress and plane strain. For plane stress, we assume the following stresses to be zero:

Stress/ Strain relationship:

The Hooke’s law relationship,

σ=Eϵ

For 2D,

ϵxx=σxxE-υσyyE

ϵyy=-υσxxE+σyyE

γxy=τxyG=21+υEτxy

Where the shear modulus G is defined as,

G=E21+υ

ϵxxϵyyγxy=1E1-υ0-υ100021+υσxxσyyτxy

σxxσyyτxy=E1-υ21υ0υ10001-υ2ϵxxϵyyγxy

σxxσyyτxy=E1-υ21υ0υ10001-υ2∂u∂x∂v∂y∂u∂y+∂v∂x

Without loss of generality, For orthotropic plates, the stress strain relationship can be written as

σxxσyyτxy=A11A120A21A22000A33ϵxxϵyyγxy

σxxσyyτxy=Ex1-υxυyυxEx1-υxυy0υyEy1-υxυyEy1-υxυy000ExEy21+υxυyϵxxϵyyγxy

Where the Ex and Ey are Young’s moduli in x and y directions, respectively, vx and vy are Poisson’s ratios. Note that the Betti principle vxEx = vyEy give A12 = A21.

\subsection{Equilibrium Equation of }

\begin{figure}[H]

\centering

\includegraphics[width=0.8\textwidth]{circular_plate_inplane_vibration_pictures/ pic3_stressandbodyforces.png}

\caption{Stresses and body forces that act on a plane differential element of uniform thickness.}

\label{fig:fig3_controlvolume_stress}

\end{figure}

The equilibrium equation for free in-plane vibration in the rectangular coordinate system shown in \fig{fig3_controlvolume_stress } can be written as,

For simplicity, we initially consider the equilibrium of a plane element subjected to normal stresses sx and sy, in-plane shear stress txy (in units of force per unit area), and body forces Xb and Yb (in units of force per unit volume), as shown in Figure above. The stresses are assumed to be constant as they act on the width of each face. However, the stresses are assumed to vary from one face to the opposite. For example, we have sx acting on the left vertical face, whereas sx þ ðqsx=qxÞ dx acts on the right vertical face. The element is assumed to have unit thickness.

Summing forces in the x direction, we have

In, X direction:

σxx+∂σxx∂xdxdy-σxxdy+τyx+∂τyx∂ydydx-τyxdx+∂2u∂t2ρdxdy=0

∂σxx∂xdxdy+∂τyx∂ydxdy+∂2u∂t2ρdxdy=0

∂σxx∂x+∂τxy∂y+∂2u∂t2ρ=0

In, Y direction:

∂σyy∂y+∂τxy∂x+∂2v∂t2ρ=0

The equilibrium equations will also be considered in the matrix form. If you consider the transpose of the symmetric gradient operator given in previous section and the column matrix form of the stress:

∇T=∂∂x0∂∂y0∂∂y∂∂x

σ=σxxσyyτxy

M=ρ∂2∂t2uv

Then the matrix form of equilibrium equation can be written as

M+∇T σ=0

The constitutive equation can be written as

σxxσyyτxy=E1-υ21υ0υ10001-υ2∂u∂x∂v∂y∂u∂y+∂v∂x

σxxσyyτxy=E1-υ21υ0υ10001-υ2∂∂x00∂∂y∂∂y∂∂xuv

σ=D∇u

Where,

D=E1-υ21υ0υ10001-υ2

In a simplified form the equation of motion can be written as,

ρ∂2∂t2+∇T D∇uv=0

The in-plane normal and shear stresses are given by

σxx=E1-υ2∂u∂x+υ∂v∂y

σyy=E1-υ2υ∂u∂x+∂v∂y

τxy=E1-υ21-υ2∂u∂y+∂v∂x

Substituting in the equation of motion gives:

E1-υ2∂2u∂x2+υ∂2v∂x∂y+E1-υ21-υ2∂2u∂y2+∂2v∂x∂y+∂2u∂t2ρ=0

E1-υ2∂2u∂x2+Eυ1-υ2∂2v∂x∂y+E1-υ21-υ2∂2u∂y2+E1-υ21-υ2∂2v∂x∂y+∂2u∂t2ρ=0

E1-υ2∂2u∂x2+E1-υ21+υ2∂2v∂x∂y+E1-υ21-υ2∂2u∂y2+∂2u∂t2ρ=0

E1-υ2∂2u∂x2+E21-υ∂2v∂x∂y+E21+υ∂2u∂y2+∂2u∂t2ρ=0

In Y-direction:

E21+υ∂2v∂x2+E21-υ∂2u∂x∂y+E1-υ2∂2v∂y2+∂2v∂t2ρ=0

In matrix form:

ρ∂2∂t21001uv+∂2∂x2E1-υ200E21+υuv+E21-υ∂2∂x∂y0110uv+∂2∂y2E21+υ00E1-υ2uv=0

\subsubsection{Basic relations in polar coordinates}

The polar coordinate system (r, θ) and the cartesian system (x, y) are related by the following expressions:

x=rcosθ

y=rsinθ

r2=x2+y2

θ=tan-1yx

These equations yield:

∂r∂x=xr=cosθ

∂r∂y=yr=sinθ

∂θ∂x=-yr2=-sinθr

∂θ∂y=xr2=cosθr

Any derivatives with respect to x and y in the Cartesian system may be transformed into derivatives with respect to r and θ by applying the chain rule:

∂∂x=∂r∂x∂∂r+∂θ∂x∂∂θ=cosθ∂∂r-sinθr∂∂θ

∂∂y=∂r∂y∂∂r+∂θ∂y∂∂θ=sinθ∂∂r+cosθr∂∂θ

\subsection {Strain displacement relations in polar coordinates}

Consider now the deformation of the infinitesimal element abcd, denoting the r and θ displacements by u_r and u_θ , respectively. The general deformation experienced by an element may be regarded as composed of (1) a change in length of the sides, and (2) rotation of the sides, as in Figure \ref { fig4_ controlvolume_deform_polarcoordinates }.

\begin{figure}[H]

\centering

\includegraphics[width=1.0\textwidth]{circular_plate_inplane_vibration_pictures/pic4_ deformpolarcoordinates.png}

\caption{Deformation of a infinitesimal 2D control volume showing: (a) linear and angular along the radial axis and (b) linear and angular changes in angular axis.}

\label{fig:fig4_controlvolume_deform_polarcoordinates}

\end{figure}

Let the continuous displacement field at an arbitrary point $A(r, θ)$ be represented by the scalar components $u_r(r, θ)$ and $u_θ (r, θ)$ along the $r radial$- and $θ angular$-directions, respectively. By applying a first-order Taylor series expansion, the linear and angular displacements of the adjacent boundaries of the infinitesimal element ($dr \times dθ$) can be expressed as:

urr+dr,θ=urr,θ+∂ur∂rdr

urr,θ+dθ=urr,θ+∂ur∂θdθ

uθr,θ+dθ=uθr,θ+∂uθ∂θdθ

uθr+dr,θ=uθr,θ+∂uθ∂rdr

Unlike the rectangular coordinates, the polar coordinate strain relationship need the analysis of strain required studying the deformation in both radial and angular for the particular component.  In the analysis that follows, the small angle approximation sin is employed, and arcs ab and cd are regarded as straight lines. Referring to Figure \fig{fig4_controlvolume_deform_polarcoordinates }, it is observed that a u_r displacement of side ab results in both radial and tangential strain. The radial strain ε_r the deformation per unit length of side ad, is associated only with the u_r displacement:

ϵr=urr,θ+∂ur∂rdr-urr,θdr=∂ur∂r

The tangential strain owing to u_r, the deformation per unit length of ab, is

ϵθur=r+urdθ-rdθrdθ=urr

Clearly, a u_θ displacement of element abcd also produces a tangential strain,

ϵθuθ=uθr,θ+∂uθ∂θdθ-uθr,θrdθ=1r∂uθ∂θ

since the increase in length of ab is (du_θ /dθ) dθ The resultant tangential strain, combining Eqs. (d) and (e), is

ϵθ=ϵθur+ϵθuθ=1r∂uθ∂θ+urr

The rotation of side bc associated with a displacement alone is shown in Fig \ref{ fig5_controlvolume_shearstrain_polarcoordinates}}. The figure clearly shows the relative rotation is not the same as rectangular coordinate but instead the initial rotation need to account for.  Since an initial rotation of through an angle u_θ /r has occurred, the relative rotation of side bc is

\begin{figure}[H]

\centering

\includegraphics[width=1.0\textwidth]{circular_plate_inplane_vibration_pictures/pic5_sheardeformpolarcoordinates.png}

\caption{Deformation of a infinitesimal 2D control volume showing associated shear strain}

\label{fig:fig5_controlvolume_shearstrain_polarcoordinates}

\end{figure}

γrθuθ=∂uθ∂r-uθr

γrθur=ur+∂ur∂θdθ-urr dθ=1r∂ur∂θ

The sum of the equations provides the total shear strain.

γrθ=γrθur+γrθuθ=∂uθ∂r+1r∂ur∂θ-uθr

In the matrix operator format for 2D case, the strain – displacement relation are

ϵrϵθγrθ=∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1ruruθ

\subsubsubsection{Stress/ Strain relationship in Polar Coordinates:}

To write Hooke’s law in polar coordinates, we need only replace subscripts x by r and y by θ in the appropriate Cartesian equations. σ=Eϵ

For 2D,

ϵr=σrE-υσθE

ϵθ=-υσrE+σθE

γrθ=τrθG=21+υEτrθ

Where the shear modulus G is defined as,

G=E21+υ

ϵrϵθγrθ=1E1-υ0-υ100021+υσrσθτrθ

σrσθτrθ=E1-υ21υ0υ10001-υ2ϵrϵθγrθ

σrσθτrθ=E1-υ21υ0υ10001-υ2∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1ruruθ

σ=D∇u

Where,

D=E1-υ21υ0υ10001-υ2

\subsubsection{Equations of equilibrium in polar coordinates}

Consider the state of stress on an infinitesimal element abcd of unit thickness de-scribed by polar coordinates in figure \ref{ fig5_stresselement_polarcoordinates }. The r and θ directed body forces are de-noted by F_r and F_θ Equilibrium of radial forces requires that

\begin{figure}[H]

\centering

\includegraphics[width=1.0\textwidth]{circular_plate_inplane_vibration_pictures/pic5_ stresspolarcoordinates.png}

\caption{Stress element in polar coordinates.}

\label{fig:fig5_stresselement_polarcoordinates}

\end{figure}

σr+∂σr∂rdrr+drdθ-σrrdθ-σθ+∂σθ∂θdθdrsindθ2-σθdrsindθ2+τrθ+∂τrθ∂θdθdrcosdθ2-τrθdrcosdθ2-∂2ur∂t2ρdrdθ =0

Inasmuch as dθ is small, sin(dθ/2) may be replaced by (dθ/2) and cos(dθ/2) by 1.

σr+∂σr∂rdrr+drdθ-σrrdθ-σθ+∂σθ∂θdθdrdθ2-σθdrdθ2+τrθ+∂τrθ∂θdθdr-τrθdr-∂2ur∂t2ρdrdθ =0

σrdrdθ+∂σr∂rrdrdθ+∂σr∂rdθdr2-σθdrdθ-∂σθ∂θdrdθ22+∂τrθ∂θdθdr-∂2ur∂t2ρdrdθ =0

The higher order terms of infinitesimal can be removed.

σrdrdθ+∂σr∂rrdrdθ-σθdrdθ+∂τrθ∂θdθdr-∂2ur∂t2ρdrdθ =0

Dividing by rdrdθ

∂σr∂r+1r∂τrθ∂θ+σr-σθr =∂2ur∂t2ρr

The equilibrium equation in the other direction.

τrθ+∂τrθ∂rdrr+drdθ-τrθrdθ+σθ+∂σθ∂θdθdrcosdθ2-σθdrcosdθ2-τrθ+∂τrθ∂θdθdrsindθ2-τrθdrsindθ2-∂2uθ∂t2ρdrdθ =0

Inasmuch as dθ is small, sin(dθ/2) may be replaced by (dθ/2) and cos(dθ/2) by 1.

τrθ+∂τrθ∂rdrr+drdθ-τrθrdθ+σθ+∂σθ∂θdθdr-σθdr+τrθ+∂τrθ∂θdθdrdθ2+τrθdrdθ2-∂2uθ∂t2ρdrdθ =0

τrθrdθ+∂τrθ∂rrdrdθ+τrθdrdθ+∂τrθ∂rdr2dθ-τrθrdθ+σθdr+∂σθ∂θdrdθ-σθdr+τrθdrdθ2+∂τrθ∂θdrdθ22+τrθdrdθ2-∂2uθ∂t2ρdrdθ =0

∂τrθ∂rrdrdθ+τrθdrdθ+∂τrθ∂rdr2dθ+∂σθ∂θdrdθ+τrθdrdθ+∂τrθ∂θdrdθ22-∂2uθ∂t2ρdrdθ =0

The higher order terms of infinitesimal can be removed and dividing by rdrdθ.

∂τrθ∂r+1r∂σθ∂θ+2τrθr =∂2uθ∂t2ρr

\subsubsection{Matrix Operator Representation}

To facilitate numerical discretization and compact analytical handling, the coupled elastodynamic partial differential equations are expressed in a generalized matrix-operator format. We define the symmetric strain-displacement gradient operator vector $\mathbf{\nabla}$ and its divergence transpose $\mathbf{\nabla}^T$ as:

∂σr∂r+1r∂τrθ∂θ+σr-σθr =∂2ur∂t2ρr

∂τrθ∂r+1r∂σθ∂θ+2τrθr =∂2uθ∂t2ρr

∇polar=∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1r

∇st=∂∂r+1r-1r1r∂∂θ01r∂∂θ∂∂r+2r

∇stσ=ρr∂2∂t2uruθ

∇stD∇polaruruθ-ρr∂2∂t2uruθ=0

Let the internal stress fields be grouped into their respective column matrices:

σ=σrσθτrθ

Analytic solution of Navier – Cauchy equation

Helmholtz Decomposition Theorem:

Helmholtz decomposition theorem states that any vector filed can be decomposed to purely diverging and purely solenoidal. For the polar displacement vector field u_r and u_θ

u=∇uϕ+∇u×ψez

This splits the motion into longitudinal/ pressure wave potential (dilatational) φ and shear (rotational) ψ.

∇uϕ=∂∂r1r∂∂θϕ

∇u×ψez=ijk∂∂r1r∂∂θ000ezψ=1r∂ψ∂θi-∂ψ∂rj=1r∂∂θ-∂∂rψ

Now the vector field can be written as,

uruθ=∂∂r1r∂∂θϕ+1r∂∂θ-∂∂rψ

$\phi(r,\theta,t)$ represents the dilatational (longitudinal/pressure) wave potential. $\psi(r,\theta,t)$ represents the rotational (transverse/shear) wave potential.

\Paragraph (Expanded Navier – Cauchy Polar Operator)

E1-ν2∂∂r+1r-1r1r∂∂θ01r∂∂θ∂∂r+2r1ν0ν10001-ν2∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1ruruθ=ρ∂2∂t2uruθ

E1-ν2∂∂r+1r-1r1r∂∂θ01r∂∂θ∂∂r+2r1ν0ν10001-ν2∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1ruruθ=ρ∂2∂t2uruθ

E1-ν2∂∂r+1r-1r1r∂∂θ01r∂∂θ∂∂r+2r∂∂r+νrνr∂∂θν∂∂r+1r1r∂∂θ1-ν21r∂∂θ1-ν2∂∂r-1ruruθ=ρ∂2∂t2uruθ

Term 1:

∂∂r+1r∂∂r+νr-1rν∂∂r+1r+1r∂∂θ1-ν21r∂∂θ

∂∂r∂∂r+νr+1r∂∂r+νr-νr∂∂r-1r2+1-ν2r2∂2∂θ2

∂2∂r2+νr∂∂r-νr2+1r∂∂r+νr2-νr∂∂r-1r2+1-ν2r2∂2∂θ2

∂2∂r2+1r∂∂r-1r2+1-ν2r2∂2∂θ2

Term 2:

∂∂r+1rνr∂∂θ-1r1r∂∂θ+1r∂∂θ1-ν2∂∂r-1r

∂∂rνr∂∂θ+vr2∂∂θ-1r2∂∂θ+1-ν2r∂2∂r∂θ-∂∂θ1r

∂∂rνr∂∂θ+vr2∂∂θ-1r2∂∂θ+1-ν2r∂2∂r∂θ-1r∂∂θ

νr∂2∂r∂θ-vr2∂∂θ+vr2∂∂θ-1r2∂∂θ+1-ν2r∂2∂r∂θ+ν-12r2∂∂θ

1+ν2r∂2∂r∂θ+ν-32r2∂∂θ

Term 3:

1r∂∂θν∂∂r+1r+∂∂r+2r1-ν21r∂∂θ

vr∂2∂θ∂r+1r2∂∂θ+1-ν2∂∂r1r∂∂θ+2r1r∂∂θ

vr∂2∂θ∂r+1r2∂∂θ+1-ν21r∂2∂θ∂r-1r2∂∂θ+2r2∂∂θ

vr∂2∂θ∂r+1r2∂∂θ+1-ν2r∂2∂θ∂r+1-ν2r2∂∂θ

1+ν2r∂2∂θ∂r+3-ν2r2∂∂θ

Term 4:

1r∂∂θ1r∂∂θ+1-ν2∂∂r+2r∂∂r-1r

1r2∂2∂θ2+1-ν2∂∂r∂∂r-1r+2r∂∂r-1r

1r2∂2∂θ2+1-ν2∂2∂r2-1r∂∂r+1r2+2r∂∂r-2r2

1r2∂2∂θ2+1-ν2∂2∂r2+1r∂∂r-1r2

1-ν2∂2∂r2+1-ν2r∂∂r+v-12r2+1r2∂2∂θ2

Combining all the terms to the matrix form gives

E1-ν2∂2∂r2+1r∂∂r-1r2+1-ν2r2∂2∂θ21+ν2r∂2∂r∂θ+ν-32r2∂∂θ1+ν2r∂2∂θ∂r+3-ν2r2∂∂θ1-ν2∂2∂r2+1-ν2r∂∂r+v-12r2+1r2∂2∂θ2uruθ=ρ∂2∂t2uruθ

\paragraph{Unified Open System Operator Layout}

Reassembling the four calculated differential components back into the main matrix structure establishes the complete, expanded Navier-Cauchy system equations for 2D polar space:

\begin{equation}

\frac{E}{1-\nu^2} \begin{bmatrix} \dfrac{\partial^2}{\partial r^2} + \dfrac{1}{r}\dfrac{\partial}{\partial r} - \dfrac{1}{r^2} + \dfrac{1-\nu}{2r^2}\dfrac{\partial^2}{\partial \theta^2} & \dfrac{1+\nu}{2r}\dfrac{\partial^2}{\partial r \partial \theta} + \dfrac{\nu-3}{2r^2}\dfrac{\partial}{\partial \theta} \\ \dfrac{1+\nu}{2r}\dfrac{\partial^2}{\partial \theta \partial r} + \dfrac{3-\nu}{2r^2}\dfrac{\partial}{\partial \theta} & \dfrac{1-\nu}{2}\dfrac{\partial^2}{\partial r^2} + \dfrac{1-\nu}{2r}\dfrac{\partial}{\partial r} - \dfrac{1-\nu}{2r^2} + \dfrac{1}{r^2}\dfrac{\partial^2}{\partial \theta^2} \end{bmatrix} \begin{bmatrix} u_r \\ u_\theta \end{bmatrix} = \rho \begin{bmatrix} \dfrac{\partial^2 u_r}{\partial t^2} \\ \dfrac{\partial^2 u_\theta}{\partial t^2} \end{bmatrix}

\end{equation}

\label{sec:navier_expanded_polar}

Radial part:

E1-ν2∂2ur∂r2+1r∂ur∂r-urr2+1-ν2r2∂2ur∂θ2+1+ν2r∂2uθ∂r∂θ+ν-32r2∂uθ∂θ=ρ∂2ur∂t2

Rotation part:

E1-ν21+ν2r∂2ur∂θ∂r+3-ν2r2∂ur∂θ+1-ν2∂2uθ∂r2+1-ν2r∂uθ∂r+v-1uθ2r2+1r2∂2uθ∂θ2=ρ∂2uθ∂t2

Introducing the target quantities

Divergence:

∆=∂ur∂r+urr+1r∂uθ∂θ

Rotation:

2ωz=∂uθ∂r+uθr-1r∂ur∂θ

Differentiate the divergence and rotation term:

∂∆∂r=∂2ur∂r2+1r∂ur∂r-urr2+1r∂2uθ∂r∂θ-1r2∂uθ∂θ

∂ωz∂θ=12∂2uθ∂θ∂r+12r∂uθ∂θ-12r∂2ur∂θ2

∂∆∂θ=∂2ur∂r∂θ+1r∂ur∂θ+1r∂2uθ∂θ2

∂ωz∂r=12∂2uθ∂r2+12r∂uθ∂r-uθ2r2-12r∂2ur∂r∂θ+12r2∂ur∂θ

Define the elastic constants (Lame’s parameter),

λ=Eν1+ν1-ν

G=E21+ν

Combining the term strategically to obtain the radial part:

λ+2G∂∆∂r-2Gr∂ωz∂θ=λ+2G∂∆∂r-2Grλ+2G∂ωz∂θ

λ+2G∂2ur∂r2+1r∂ur∂r-urr2+1r∂2uθ∂r∂θ-1r2∂uθ∂θ-Grλ+2G∂2uθ∂θ∂r-Gr2λ+2G∂uθ∂θ+Gr2λ+2G∂2ur∂θ2

λ+2G∂2ur∂r2+1r∂ur∂r-urr2+λ+2Grλ+2G∂2uθ∂r∂θ-λ+2Gr2λ+2G∂uθ∂θ-Grλ+2G∂2uθ∂θ∂r-Gr2λ+2G∂uθ∂θ+Gr2λ+2G∂2ur∂θ2

λ+2G∂2ur∂r2+1r∂ur∂r-urr2+λ+Grλ+2G∂2uθ∂θ∂r-λ+3Gr2λ+2G∂uθ∂θ+Gr2λ+2G∂2ur∂θ2

=E1-ν2∂2ur∂r2+1r∂ur∂r-urr2+1-ν2r2∂2ur∂θ2+1+ν2r∂2uθ∂r∂θ+ν-32r2∂uθ∂θ

λ+2G=Eν1+ν1-ν+E1-ν1+ν1-ν=E1-ν2

λ+G=2Eν21+ν1-ν+E1-ν21+ν1-ν=E1+ν21-ν2

λ+3G=2Eν21+ν1-ν+3E1-ν21+ν1-ν=E3-ν21-ν2

λ+Gλ+2G=1+ν2

λ+3Gλ+2G=3-ν2

Gλ+2G=1-ν2EE21+ν=1-ν2

Combining the term strategically to obtain the rotation part:

λ+2G1r∂∆∂θ+2G∂ωz∂r=λ+2G1r∂∆∂θ+2Gλ+2G∂ωz∂r

λ+2G1r∂2ur∂r∂θ+1r2∂ur∂θ+1r2∂2uθ∂θ2+Gλ+2G∂2uθ∂r2+1r∂uθ∂r-uθr2-1r∂2ur∂r∂θ+1r2∂ur∂θ

λ+2G1-Gλ+2G1r∂2ur∂r∂θ+1+Gλ+2G1r2∂ur∂θ+1r2∂2uθ∂θ2+Gλ+2G∂2uθ∂r2+1r∂uθ∂r-uθr2

λ+2Gλ+3Gλ+2G1r2∂ur∂θ+λ+Gλ+2G1r∂2ur∂r∂θ+1r2∂2uθ∂θ2+Gλ+2G∂2uθ∂r2+1r∂uθ∂r-uθr2

λ+2G3-ν2r2∂ur∂θ+1+ν2r∂2ur∂r∂θ+1r2∂2uθ∂θ2+1-ν2∂2uθ∂r2+1r∂uθ∂r-uθr2

E1-ν21+ν2r∂2ur∂θ∂r+3-ν2r2∂ur∂θ+1-ν2∂2uθ∂r2+1-ν2r∂uθ∂r+v-1uθ2r2+1r2∂2uθ∂θ2

Therfore the above derivation shows that the,

\subsection{Mathematical Decoupling of the Polar Navier-Cauchy Equations}

To perform the analytical decoupling, we first rewrite the polar equations of motion derived in the previous sections by collecting terms into the volumetric dilatation ($\Delta$) and the out-of-plane rigid rotation ($\omega_z$). This configuration reads:

\begin{equation}

(\lambda + 2G) \frac{\partial \Delta}{\partial r} - \frac{2G}{r} \frac{\partial \omega_z}{\partial \theta} = \rho \frac{\partial^2 u_r}{\partial t^2}

\label{eq:navier_decouple_r}

\end{equation}

\begin{equation}

(\lambda + 2G) \frac{1}{r} \frac{\partial \Delta}{\partial \theta} + 2G \frac{\partial \omega_z}{\partial r} = \rho \frac{\partial^2 u_\theta}{\partial t^2}

\label{eq:navier_decouple_theta}

\end{equation}

where Lamé's first parameter for plane stress is $\lambda = \frac{E\nu}{1-\nu^2}$, and the kinematic expansions of $\Delta$ and $2\omega_z$ are given by:

\begin{align}

\Delta &= \frac{\partial u_r}{\partial r} + \frac{u_r}{r} + \frac{1}{r}\frac{\partial u_\theta}{\partial \theta} \label{eq:dilatation_def} \\

2\omega_z &= \frac{\partial u_\theta}{\partial r} + \frac{u_\theta}{r} - \frac{1}{r}\frac{\partial u_r}{\partial \theta} \label{eq:rotation_def}

\end{align}

\section{Solving the Decoupled Elastodynamic wave equation in polar coordinates}

Potentials oscillate harmonically in time with a natural frequency ω

ϕr,θ,t=Φr,θeiωt

ψr,θ,t=Ψr,θeiωt

Substituting in the wave equations yields the spatial Helmholtz equations:

∇2Φ+α2Φ=0

∇2Ψ+β2Ψ=0

Where,

α2=ω2c12

β2=ω2c22

Applying the separation of variables for the spatial domain, splitting them into radial profiles (R) and angular profiles (θ)

Φr,θ=R1rΘ1θ

Ψr,θ=R2rΘ2θ

Because a full circular disk has geometric continuity around its circumference, the angular solution must be periodic (ϴ(0) = ϴ(0+2π)). This forces the angular solutions to be combinations of sines and cosines matching an integer circumferential mode number (n):

Θθ=Ancosnθ+Bnsinnθ

Solving the Radial Equations:

With the angular dependency set to an integer $n$, the remaining radial differential equations transform into Bessel's standard differential equations:

r2d2R1dr2+rdR1dr+α2r2-n2R1=0

r2d2R2dr2+rdR2dr+β2r2-n2R2=0

The general solution to these equation are linear combinations of Bessel functions of the first kind (Jn) and second kind, Neumann function (Yn)

R1r=C1Jnαr+C2Ynαr

R2r=C3Jnβr+C4Ynβr

Application of Boundary conditions:

The domain under investigation comprises a solid circular disk that includes the geometric origin ($r = 0$), therefore the Neumann functions must be suppressed ($C_2 = C_4 = 0$). This constraint arises because $Y_n(x) \to -\infty$ as $x \to 0$, which is physically inadmissible since displacements and strain energy densities must remain finite at the center of a continuous elastic body.

\subsection{Derivation of the Free Characteristic Frequency Equation}

The constitutive equations for plane-stress linear elasticity define the boundary stresses in terms of the potential fields. Substituting $\Phi$ and $\Psi$ into these constitutive relations at $r = a$ leads to a homogeneous system of equations for the coefficients $C_1$ and $C_3$:

\begin{equation}

\begin{bmatrix}

M_{11}(\alpha a) & M_{12}(\beta a) \\[6pt]

M_{21}(\alpha a) & M_{22}(\beta a)

\end{bmatrix}

\begin{bmatrix} C_1 \\ C_3 \end{bmatrix} = \begin{bmatrix} 0 \\ 0 \end{bmatrix}

\end{equation}

By evaluating the polar stress-displacement relations, the explicit matrix coefficients are derived using the recurrence properties of Bessel functions:

\begin{align}

M_{11}(\alpha a) &= \left[ 2n(n-1) - (\beta a)^2 \right] J_n(\alpha a) + 2\alpha a J_{n+1}(\alpha a) \\

M_{12}(\beta a) &= 2n(n-1) J_n(\beta a) - 2n\beta a J_{n+1}(\beta a) \\

M_{21}(\alpha a) &= 2n(n-1) J_n(\alpha a) - 2n\alpha a J_{n+1}(\alpha a) \\

M_{22}(\beta a) &= \left[ 2n(n-1) - (\beta a)^2 \right] J_n(\beta a) + 2\beta a J_{n+1}(\beta a)

\end{align}

For a non-trivial solution to exist ($C_1, C_3 \neq 0$), the determinant of the characteristic matrix must equal zero:

\begin{equation}

\det \begin{bmatrix}

M_{11}(\alpha a) & M_{12}(\beta a) \\

M_{21}(\alpha a) & M_{22}(\beta a)

\end{bmatrix} = 0 \implies M_{11}M_{22} - M_{12}M_{21} = 0

\end{equation}

Roots of this transcendental equation yield the discrete natural frequencies $\omega_{nm}$ for each circumferential mode $n$ and radial mode $m$. Solving this determinant establishes the amplitude ratio coefficient:

\begin{equation}

K_n = \frac{C_3}{C_1} = -\frac{M_{11}(\alpha a)}{M_{12}(\beta a)}

\end{equation}

\subsection{Derivation of the Free Characteristic Frequency Equation}

The constitutive equations for plane-stress linear elasticity define the boundary stresses in terms of the potential fields. Substituting $\Phi$ and $\Psi$ into these constitutive relations at $r = a$ leads to a homogeneous system of equations for the coefficients $C_1$ and $C_3$:

Potential fields

Φ=C1Jnαrcosnθ

Ψ=C3Jnβrsinnθ

Substituting in the wave equations yields the spatial Helmholtz equations:

∇2Φ+α2Φ=0

∇2Ψ+β2Ψ=0

Constitutive relation at r = a (boundary), leading to a homogeneous system for coefficients.

In polar coordinates (r, theta) the displacement components (u_r, u_theta) are given by:

ur=∂Φ∂r+1r∂ψ∂θ=C1αJn'αrcosnθ+C3nrJnβrcosnθ

uθ=1r∂Φ∂θ-∂ψ∂r=-C1nrJnαrsinnθ-C3βJn'βrsinnθ

Writing in matrix form

uruθ=α∂Jnαr∂rcosnθnrJnβrcosnθ-nrJnαrsinnθ-β∂Jnβr∂rsinnθC1C3

σ=D∇polaruruθ

σ=E1-υ21υ0υ10001-υ2∂∂r01r1r∂∂θ1r∂∂θ∂∂r-1ruruθ

σrσθτrθ=E1-υ21υ0υ10001-υ2∂ur∂rurr+1r∂uθ∂θ1r∂ur∂θ+∂uθ∂r-uθr

σr=E1-υ2∂ur∂r+υurr+1r∂uθ∂θ

σθ=E1-υ2υ∂ur∂r+urr+1r∂uθ∂θ

τrθ=E21+υ1r∂ur∂θ+∂uθ∂r-uθr

Compute derivative needed

First term

∂ur∂r=C1α2∂2Jnαr∂r2cosnθ+C3n∂∂rJnβrrcosnθ

∂∂rJnβrr=βr∂Jnβr∂r-Jnβrr2=βr∂Jnβr∂r-Jnβrr2

∂ur∂r=C1α2Jn''αrcosnθ+C3nβJn'βrr-Jnβrr2cosnθ

Second term

∂ur∂θ=C1α∂Jnαr∂r∂cosnθ∂θ+C3nrJnβr∂cosnθ∂θ

∂ur∂θ=-C1αnJn'αrsinnθ-C3n2rJnβrsinnθ

Third term

∂uθ∂θ=-C1nrJnαr∂sinnθ∂θ-C3β∂Jnβr∂r∂sinnθ∂θ

∂uθ∂θ=-C1n2rJnαrcosnθ-C3βnJn'βrcosnθ

Fourth term

∂uθ∂r=-C1n∂∂rJnαrrsinnθ-C3β2∂2Jnβr∂r2sinnθ

∂∂rJnαrr=αr∂Jnαr∂r-Jnαrr2

∂uθ∂r=-C1nαJn'αrr-Jnαrr2sinnθ-C3β2Jn''βrsinnθ

σr=E1-υ2∂ur∂r+υurr+1r∂uθ∂θ

σr=E1-υ2C1α2Jn''αrcosnθ+C3nβJn'βrr-Jnβrr2cosnθ+υC1αrJn'αrcosnθ+C3nr2Jnβrcosnθ-C1n2r2Jnαrcosnθ-C3βnrJn'βrcosnθ

σr=E1-υ2C1α2Jn''αr+C3nβJn'βrr-C3nJnβrr2+C1υαrJn'αr+C3υnr2Jnβr-C1υn2r2Jnαr-C3βυnrJn'βrcosnθ

σr=E1-υ2C1α2Jn''αr+C1υαrJn'αr-C1υn2r2Jnαr+C3nβrJn'βr-C3nr2Jnβr+C3υnr2Jnβr-C3υnβrJn'βrcosnθ

σr=E1-υ2C1α2Jn''αr+υαrJn'αr-υn2r2Jnαr+C3n1-υβrJn'βr-1r2Jnβrcosnθ

Using Helmholtz equation to remove the second order derivative

∇2Φ+α2Φ=0

∂2Φ∂r2+1r∂Φ∂r+1r2∂2Φ∂θ2+α2Φ=0

Substituting for phi

C1α2Jn''αrcosnθ+C1αrJn'αrcosnθ-n2r2C1Jnαrcosnθ+α2C1Jnαrcosnθ=0

α2Jn''αr+αrJn'αr+α2-n2r2JnαrC1cosnθ=0

α2Jn''αr+αrJn'αr+α2-n2r2Jnαr=0

Substituting,

σr=E1-υ2C1-1-υαrJn'αr-α2Jnαr+1-υn2r2Jnαr+C3n1-υβrJn'βr-1r2Jnβrcosnθ

σr=E1-υ2C1-1-υαrJn'αr+1-υn2r2-α2Jnαr+C3n1-υβrJn'βr-1r2Jnβrcosnθ

τrθ=E21+υ1r∂ur∂θ+∂uθ∂r-uθr

τrθ=E21+υ-C1αnrJn'αrsinnθ-C3n2r2Jnβrsinnθ+-C1nαJn'αrr-Jnαrr2sinnθ-C3β2Jn''βrsinnθ+C1nr2Jnαrsinnθ+C3βJn'βrrsinnθ

τrθ=E1-υ21-υ2-C12nαrJn'αr+C12nr2Jnαr-C3β2Jn''βr+C3βrJn'βr-C3n2r2Jnβrsinnθ

τrθ=E1-υ21-υ2C1-2nαrJn'αr+2nr2Jnαr+C3-β2Jn''βr+βrJn'βr-n2r2Jnβrsinnθ

τrθ=E1-υ21-υ2C1-2nαrJn'αr+2nr2Jnαr+C3βrJn'βr+β2-n2r2Jnβr+βrJn'βr-n2r2Jnβrsinnθ

τrθ=E1-υ21-υ2C1-2nαrJn'αr+2nr2Jnαr+C32βrJn'βr+β2-2n2r2Jnβrsinnθ

Substituting for free – free boundary condition at r = a,

σr=τrθ=0

E1-υ2C1-1-υαaJn'αr+n21-υa2-α2Jnαa+C3n1-υβaJn'βa-1a2Jnβa=0

E1-υ21-υ2C1-2nαaJn'αa+2na2Jnαa+C32βaJn'βa+β2-2n2a2Jnβa=0

In matrix form:

M11M12M21M22C1C3=0

M11=E1-υ2-1-υαaJn'αr+n21-υa2-α2Jnαa

M12=nE1+υβaJn'βa-1a2Jnβa

M21=nE1+υ-αaJn'αa+1a2Jnαa

M22=E1+υβaJn'βa+β22-n2a2Jnβa

For a non-trivial solution to exist ($C_1, C_3 \neq 0$), the determinant of the characteristic matrix must equal zero:

\begin{equation}

\det \begin{bmatrix}

M_{11}(\alpha a) & M_{12}(\beta a) \\

M_{21}(\alpha a) & M_{22}(\beta a)

\end{bmatrix} = 0 \implies M_{11}M_{22} - M_{12}M_{21} = 0

\end{equation}

Roots of this transcendental equation yield the discrete natural frequencies $\omega_{nm}$ for each circumferential mode $n$ and radial mode $m$. Solving this determinant establishes the amplitude ratio coefficient:

\begin{equation}

K_n = \frac{C_3}{C_1} = -\frac{M_{11}(\alpha a)}{M_{12}(\beta a)}

\end{equation}